# Technical Guide: W: Web Pipe Device  Y: Clipboard Device

Author: 13leader.net

Date: January 2026

Project Repo: https://github.com/pjones1063/AspeQt-2k26

1. Overview

## W: Web Pipe Device (Thin Client Gateway)

The W: device transforms the Atari 8-bit into a "Thin Client" capable of interacting with modern web services. Instead of implementing complex network stacks (TCP/IP, SSL) on the Atari, the W: device acts as a transparent proxy. It forwards standard BASIC text commands to a backend script running on your PC via HTTP.

### How It Works

The device operates using standard Atari I/O commands:

- Write Mode (HTTP POST): Opening W: for write allows you to buffer data. When you CLOSE the channel, AspeQt sends the buffered data as an HTTP POST request to the specified URL.

- Read Mode (HTTP GET): Opening W: for read immediately performs an HTTP GET request to the URL. Subsequent INPUT commands retrieve the server's response.

Running the Handler

To enable the W: and Y: devices, you must first load the handler into memory.

- Manual Loading: From the DOS menu, select option L (Binary Load) and type HANDLER.XEX.

- Automatic Loading: Rename HANDLER.XEX to AUTORUN.SYS on your boot disk. This ensures the network drivers load automatically every time you turn on the computer.

### Code Example: The "Hello World" Loop

This example demonstrates sending data to a Python script and reading the response.

1. Atari BASIC (The Front-end)

Basic

10 DIM U$(40), MSG$(40), REPLY$(40)

20 U$="W:http://192.168.1.5:5000/atari"

30 MSG$="HELLO FROM 1979"

40 REM --- STEP 1: SEND DATA (POST) ---

50 OPEN #1,8,0,U$       : REM Open for Write

60 PRINT #1;MSG$        : REM Send the message

70 CLOSE #1             : REM Trigger the Network Request

80 REM --- STEP 2: READ RESPONSE (GET) ---

90 OPEN #1,4,0,U$       : REM Open for Read

100 INPUT #1,REPLY$     : REM Read text from PC

110 PRINT "SERVER SAID: ";REPLY$

120 CLOSE #1

2. Python/Flask (The Backend) Requires Python installed on your host PC.

Python

from flask import Flask, request

app = Flask(__name__)

@app.route('/atari', methods=['GET', 'POST'])

def handle_atari():

if request.method == 'POST':

# Receive text from Atari

data = request.data.decode('utf-8')

print(f"Atari sent: {data}")

return "OK"

else:

# Send text to Atari

return "MESSAGE RECEIVED LOUD AND CLEAR"

if __name__ == '__main__':

app.run(host='0.0.0.0', port=5000)

## Y: Clipboard Device (System Integration)

The Y: device provides a bi-directional bridge between the Atari 8-bit and your host computer's system clipboard (Windows, macOS, or Linux). This allows for rapid code transfer and data sharing without saving files to disk.

### Usage Scenarios

- Paste Code: Copy BASIC listings from a website on your PC and paste them directly into the Atari editor.

- Export Data: Send program listings or variable data from the Atari to your PC's text editor.

### Commands & Syntax

| Action | Atari Command | Description |
| --- | --- | --- |
| Write to PC | LIST "Y:" | Sends the current BASIC program to the PC clipboard. |
| Write to PC | PRINT #1;"TEXT" | Sends specific text to the PC clipboard. |
| Read from PC | ENTER "Y:" | Types the PC clipboard content into the Atari (like keystrokes). |
| Read from PC | COPY "Y:" "E:" | Displays PC clipboard content on the screen. |

### Code Example: Reading & Writing

Writing to the PC Clipboard

Basic

10 REM Send a variable to the PC Clipboard

20 A$="Copied from Atari"

30 OPEN #1,8,0,"Y:"

40 PRINT #1;A$

50 CLOSE #1

Reading from the PC Clipboard Ensure you have copied text on your PC first.

Basic

10 DIM BUFFER$(256)

20 OPEN #1,4,0,"Y:"

30 INPUT #1,BUFFER$

40 PRINT "I found this on your clipboard:"

50 PRINT BUFFER$

60 CLOSE #1

# AspeQt-2k26: Application Examples Library for W:

This library contains a collection of "Hybrid Computing" examples that demonstrate the power of the W: Device. These examples offload heavy processing (AI, Web Scraping, SSL, Speech Synthesis) to a modern PC (Host) while using the Atari 8-bit as the frontend terminal.

## General Prerequisites

For all examples below, you must have:

- Python 3.x installed on your Host PC (Linux/Windows/macOS).

- Flask installed (pip install flask).

- The PC's IP Address configured in the BASIC .LST files (look for U$="W:http://...").

## 1. Gemini AI Chatbot

Description: Turns the Atari into a client for Google's Gemini Large Language Model. You can type natural language questions ("Who is Captain Kirk?") and receive intelligent, summarized answers on the Atari screen.

- BASIC File: GEMINI_AI.LST

- Python Script: gemini_ai.py

- Dependencies: pip install google-generativeai

- Setup: You must edit gemini_ai.py and insert your Google API Key.

- Usage: Run the Python script. Run the BASIC program. Type a prompt and wait for the AI response.

## 2. Basic Graphical Weather Station

Description: Fetches live weather data from the Open-Meteo API for a specific location (configured in Python) and displays it on the Atari using GRAPHICS 5 (4-color mode). It draws custom pixel-art icons for Sun, Clouds, Rain, or Snow.

- BASIC File: WEATHER.LST

- Python Script: weather.py

- Dependencies: pip install requests

- Usage:

  - Edit weather.py to set your Latitude/Longitude (default is Lincoln, ON).

  - Run the server.

  - Run the BASIC code to see the graphical dashboard.

## 3. Cloud Notepad (Persistence Demo)

Description: Demonstrates Reading and Writing to the PC's hard drive. You can type a note on the Atari, save it, turn the Atari off, and retrieve the note later. It acts as a "Cloud Drive" for text.

- BASIC File: CLOUD_NOTE.LST

- Python Script: cloud_note.py

- Dependencies: flask (Standard)

- Usage:

  - Select 1 to Read the current note.

  - Select 2 to Write/Overwrite the note.

  - The note is saved as atari_notes.txt on the PC.

## 4. Text-to-Speech Synthesizer

Description: The "Talking Atari." Any text typed on the Atari is sent to the PC, where it is read aloud using the Linux espeak engine. This demonstrates zero-latency media triggering.

- BASIC File: TTS_SERVER.LST

- Python Script: tts_server.py

- Dependencies:

  - Python: flask

  - Linux System: sudo apt install espeak

- Usage: Type a phrase on the Atari (e.g., "Hello World"). The PC speakers will speak the phrase immediately.

## 5. Web News Scraper

Description: Connects to a modern news website (like CBC or BBC), scrapes the HTML, strips out ads/javascript, and delivers a plain-text headline summary to the Atari.

- BASIC File: SCRAPER.LST

- Python Script: scraper.py

- Dependencies: pip install requests beautifulsoup4

- Usage: Run the program to get a scrolling list of the top 5 news headlines.

## 6. Linux Remote Execution

Description: Allows the Atari to execute shell commands on the Linux Host. It includes a safety whitelist to prevent unauthorized access.

- BASIC File: EXEC_LINUX.LST

- Python Script: exec_linux.py

- Dependencies: flask (Uses standard subprocess library)

- Usage: The BASIC program sends a pre-defined command (like ls -la or whoami). The PC executes it and returns the terminal output to the Atari.

## 7. Windows Remote Control

Description: Similar to the Linux version but tailored for Windows Hosts. It can launch GUI applications (Calculator, Notepad) or open URLs in the default browser.

- BASIC File: EXEC_WIN.LST

- Python Script: exec_win.py

- Dependencies: flask (Uses standard subprocess library)

- Usage: Select an app from the menu (e.g., "1. Calculator"). The app will launch on the Windows PC screen.

## 8. File Upload / Telemetry

Description: A utility to upload raw data or text files from the Atari to the PC. This can be used for saving game high scores, sensor logs, or telemetry data.

- BASIC File: FILE_UPLOAD.LST

- Python Script: file_upload.py

- Dependencies: flask

- Usage: The BASIC program simulates a file upload. The data is saved to a timestamped file in the uploads/ folder on the PC.

### Quick Dependency Install Command

To run all examples in this library, install the following packages on your Host PC:

Linux/Mac:

Bash

sudo apt install espeak

pip install flask requests beautifulsoup4 google-generativeai psutil

Windows:

PowerShell

pip install flask requests beautifulsoup4 google-generativeai psutil